# Task 5 — Power Budget & Battery Selection

LMS / Meshmerize robot, system as in the Task 4 BOM

Result: the robot draws about 0.57 A in a typical run and at most about 2.22 A if both motors stall (3.42 A with a pessimistic motor). The 2S 1000 mAh 30C LiPo runs it for about 78 minutes per charge, while one 5.5-minute match uses only about 53 mAh (5% of the pack). The pack can deliver 30 A, about 9× the worst case.

# 1. System and assumptions

Power path: 2S LiPo → XT60 → 3 A fuse → switch. The battery feeds the TB6612FNG motor supply (VM) directly, and feeds an MP1584 buck set to 5.0 V for the ESP32's 5V pin. The ESP32 board's regulator then makes 3.3 V for the ESP32, the Pololu QTRX-MD-08RC sensor, the TB6612FNG logic and the LED.

- Battery voltage: 7.4 V nominal (8.4 V full, 6.0 V empty). Totals use 7.4 V.

- Buck efficiency η = 85% (assumption; switching regulators of this type are typically 85–90%).

- 3.3 V loads pass through a linear regulator, so its input current equals its output current plus its own quiescent current. Every 3.3 V load is therefore also a 5 V load at the same current.

- Motor current is counted at the battery as if PWM were 100%. PWM (capped at about 71%) actually lowers the average battery current, so this is a safe overestimate.

- Derating factor 0.75 for runtime (from the task brief): batteries never deliver their full label capacity, and a LiPo should not be drained below about 3.0 V per cell.

- Figures marked (est.) are estimates to be replaced by multimeter measurements once the robot is built.

# 2. Current draw per component

Idle = powered on, motors stopped. Running = following the line. Peak = Wi-Fi transmitting, LED on and motors stalled.

| Component | Rail | Idle (mA) | Running (mA) | Peak / stall (mA) | Basis |
| --- | --- | --- | --- | --- | --- |
| ESP32 chip | 3.3 V | 68 | 68 (Wi-Fi off) / 100 (Wi-Fi on) | 240 | Espressif figures via JLCPCB: CPU 240 MHz radio off 30–68 mA; Wi-Fi receive 95–100 mA; Wi-Fi transmit peak 240 mA |
| ESP32 board overhead | 3.3 V / 5 V | 8 | 8 | 8 | Regulator quiescent ~5 mA, power LED ~1.3 mA, USB chip ~1.3 mA (JLCPCB) |
| Pololu QTRX-MD-08RC | 3.3 V | 17 | 17 | 17 | Maximum board current, all LEDs on (MG Super Labs) |
| TB6612FNG logic (VCC) | 3.3 V | 1.1 | 1.1 | 1.8 | ICC 1.1 mA typ, 1.8 mA max at 3 V (Toshiba p. 5) |
| Red end-zone LED | 3.3 V | 0 | 0 | 3.9 | (3.3 − 2.0 V) / 330 Ω (est. LED drop 2.0 V) |
| 5 V rail total | 5 V | 94.1 | 94.1 / 126.1 | 270.7 | Sum of the rows above |
| MP1584 buck → battery side | battery | 75 | 75 / 100 | 215 | 5 V × I₅ / (0.85 × 7.4 V) |
| Left N20 motor | battery | 0 | 250 (est.) | 1,000 | Stall 1 A for a GA12-N20 6 V 300 RPM (Handsontec); running is an estimate; to be measured |
| Right N20 motor | battery | 0 | 250 (est.) | 1,000 | Same as left |
| TB6612FNG motor side | battery | ≈ 0 | ≈ 0 | ≈ 0 | Standby current ≤ 1 µA; operating overhead not specified, negligible next to the motors |

## Converting 5 V loads to battery current

Power is conserved through the buck converter apart from its losses, so:

I_battery = (5 V × I_5V) / (η × V_battery)

Example, idle: (5 × 0.0941 A) / (0.85 × 7.4 V) = 0.4705 W / 6.29 V = 0.075 A. The battery supplies less current than the 5 V rail draws because its voltage is higher.

# 3. Totals at the battery

| Scenario | Battery current | What it represents |
| --- | --- | --- |
| Idle | 0.075 A | Powered on, calibrated, waiting for the start button |
| Typical run (competition, Wi-Fi off) | 0.575 A | Logic 0.075 A + two motors at 0.25 A |
| Practice run with Wi-Fi telemetry | 0.600 A | Wi-Fi adds about 25 mA on average |
| Worst case: both motors stalled + Wi-Fi transmitting | 2.215 A | 2 × 1.0 A stall + 0.215 A logic peak |
| Upper bound (Pololu high-power motor, 1.6 A stall) | 3.415 A | Pessimistic case using Pololu 6 V HP figures |

At the empty-battery voltage of 6.0 V the logic draws slightly more battery current (about 0.27 A peak instead of 0.22 A), because the buck must pull more current from a lower voltage. This changes the worst case by only about 0.05 A.

# 4. Runtime

Using the formula from the task brief:

Runtime (h) = Capacity (mAh) / Average current (mA) × 0.75

| Scenario | Current | Runtime | Working |
| --- | --- | --- | --- |
| Typical competition run | 575 mA | 1.30 h ≈ 78 min | 1000 / 575 × 0.75 |
| Practice with Wi-Fi telemetry | 600 mA | 1.25 h ≈ 75 min | 1000 / 600 × 0.75 |
| Idle (waiting) | 75 mA | 10.03 h | 1000 / 75 × 0.75 |
| Continuous double stall (never happens in practice) | 2215 mA | 0.34 h ≈ 20 min | 1000 / 2215 × 0.75 |

Energy cross-check: the pack stores 7.4 V × 1.0 Ah = 7.4 Wh. A typical run uses 7.4 V × 0.575 A = 4.25 W, so 7.4 Wh × 0.75 / 4.25 W = 1.30 h, which matches.

What it means: one Meshmerize match (3 min dry run + 2.5 min actual run) uses about 53 mAh. Capacity is never the limit. What matters is starting every match on a fully charged pack, so that the motors see the same voltage and the tuned PID behaves the same.

# 5. Checks against component ratings

| Check | Demand | Rating | Result |
| --- | --- | --- | --- |
| Battery maximum discharge (1.0 Ah × 30C) | 3.42 A | 30 A | OK, about 9× margin |
| Fuse | 2.22 A | 3 A | OK; it blows only on a short (or a 1.6 A-per-motor double stall) |
| TB6612FNG per channel | 1.0 A stall | 1.0 A operating, 1.2 A absolute max | OK; firmware stall timeout as backup (Task 2) |
| MP1584 output | 271 mA | 3 A | OK, large margin |
| ESP32 board regulator heat | (5 − 3.3) V × 271 mA | — | 0.46 W peak, 0.16 W typical: fine |

# 6. Battery chemistry: LiPo vs Li-ion (18650)

Both options give a nominal 7.4 V pack (2 cells in series), which suits the TB6612FNG and the 6 V motors.

| Factor | 2S LiPo, 1000 mAh 30C (chosen) | 2S Li-ion, 2 × Samsung 25R 18650 |
| --- | --- | --- |
| Capacity / energy | 1000 mAh, 7.4 Wh | 2500 mAh, 18 Wh (3.6 V × 2.5 Ah × 2) |
| Weight | ≈ 57 g (comparable 2S 1000 mAh 30C pack, Overlander) | 2 × 45 g = 90 g, plus a holder (~20 g, est.) ≈ 110 g (18650 Battery Store) |
| Energy density | 7.4 Wh / 0.057 kg ≈ 130 Wh/kg | 9.0 Wh / 0.045 kg ≈ 200 Wh/kg per cell; ≈ 164 Wh/kg with holder |
| Maximum continuous current | 30C × 1 Ah = 30 A | 20 A per cell (≈ 8C) |
| Voltage under load | Highest of the lithium types: least sag when motors surge | Sags more under high current |
| Cost (India) | ₹1,099 (Robu, 21 Oct) | ≈ ₹840 for two cells (US$4.99 each, converted, est.) + ₹59 holder (Probots) |
| Safety | Soft pouch: can swell or catch fire if punctured or over-discharged | Steel can: tougher against crashes |
| Cycle life | Lower: about half that of Li-ion | Higher |
| Fit on a 130 × 150 mm robot | Flat 56 × 30 × 19 mm pack sits low over the axle | Two 65 mm cylinders: taller and harder to place low |

Qualitative points (voltage under load, safety, cycle life) follow Unmanned Systems Technology; C-rate meaning follows Battery University BU-402.

## Why we chose the LiPo

- Weight: about 57 g against about 110 g. On a robot that brakes and pivots at junctions about 30 cm apart, half the battery weight means faster stops and more accurate turns.

- Current margin and voltage stability: 30 A available against a 3.4 A worst case, with the least voltage sag. Steadier motor voltage keeps the PID tuning consistent.

- Energy density is not the deciding factor. Li-ion stores more energy per kg, but we need only about 53 mAh per match. Extra capacity is just extra weight we would carry.

- Cost is similar: ₹1,099 against about ₹900 for cells and a holder. The ~₹200 saving doesn't outweigh the weight.

- Packaging: the flat pack mounts low over the drive axle, keeping the centre of mass low and the weight on the drive wheels.

Accepted trade-off and how we handle it: LiPo is less robust and has fewer cycles. We charge only with the balance charger inside the fire-safe bag, never discharge below 3.0 V per cell (checked with a multimeter before runs), store at about 3.8 V per cell, and retire any pack that swells.